Lab Task 1: Design an IP Address Scheme site24x7

- The network 172.16.10.0/16 was divided into seven subnets, as outlined below:

- The value of the new subnet mask is 255.255.224.0

- 2^19=8190 usable hosts exist per subnet.

| Subnet ID | Subnet Address | Host Address Range | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 172.16.0.0 | 172.16.0.1 - 172.16.31.254 | 172.16.31.255 |
| 2 | 172.16.32.0 | 172.16.32.1 - 172.16.63.254 | 172.16.63.255 |
| 3 | 172.16.64.0 | 172.16.64.1 - 172.16.95.254 | 172.16.95.255 |
| 4 | 172.16.96.0 | 172.16.96.1 - 172.16.127.254 | 172.16.127.255 |
| 5 | 172.16.128.0 | 172.16.128.1 - 172.16.159.254 | 172.16.159.255 |
| 6 | 172.16.160.0 | 172.16.160.1 - 172.16.191.254 | 172.16.191.255 |
| 7 | 172.16.192.0 | 172.16.192.1 - 172.16.223.254 | 172.16.223.255 |

Lab Task 2: Implement VLANs and Trunk
(Listed commands were executed on S1-Office1 and S2-Office1.)

- en
conf t
vlan 10
name Management
exit
vlan 20
name Marketing
exit
vlan 30
name Accounting
exit
vlan 100
name Native
exit

- int range fa0/1-10
switchport mode access
switchport access vlan 10
exit
int range fa0/11-20
switchport mode access
switchport access vlan 20
exit
int range fa0/21-24
switchport mode access
switchport access vlan 30
exit

  - On S1-Office1:

int gi0/2
switchport mode trunk
switchport trunk native vlan 100
exit
show vlan brief
show int trunk

  - On S2-Office1:

int gi0/1
switchport mode trunk
switchport trunk native vlan 100
exit
show vlan brief 
show int trunk

- int range fa0/1-24
switchport nonegotiate
exit
wr

S1-Office1# show vlan brief 					S2-Office1# show vlan brief

S1-Office1#show int trunk 						S2-Office1#show int trunk

Lab Task 3: Assign IP Addresses

| Device | Interface | Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Gig0/0/0 | --- | 255.255.224.0 | --- |
|  | Serial0/1/0 | 172.16.128.1 | 255.255.224.0 | --- |
|  | Serial0/1/1 | 172.16.160.1 | 255.255.224.0 | --- |
|  | Gig0/0/0.10 | 172.16.0.1 | 255.255.224.0 | --- |
|  | Gig0/0/0.20 | 172.16.32.1 | 255.255.224.0 | --- |
|  | Gig0/0/0.30 | 172.16.64.1 | 255.255.224.0 | --- |
| R2-Edge_router | Serial0/1/0 | 172.16.96.2 | 255.255.224.0 | --- |
|  | Serial0/1/1 | 172.16.160.2 | 255.255.224.0 | --- |
| R3 | Gig0/0/0 | 172.16.192.1 | 255.255.224.0 | --- |
|  | Serial0/1/0 | 172.16.128.2 | 255.255.224.0 | --- |
|  | Serial0/1/1 | 172.16.96.1 | 255.255.224.0 | --- |
| S1-Office1 | VLAN 10 (Management) | --- | 255.255.224.0 | 172.16.32.1 |
|  | VLAN 20 (Marketing) | --- | 255.255.224.0 | 172.16.32.1 |
|  | VLAN 30 (Accounting) | --- | 255.255.224.0 | 172.16.32.1 |
| S2-Office1 | VLAN 10 (Management) | --- | 255.255.224.0 | 172.16.160.1 |
|  | VLAN 20 (Marketing) | --- | 255.255.224.0 | 172.16.160.1 |
|  | VLAN 30 (Accounting) | --- | 255.255.224.0 | 172.16.160.1 |
| CEO1 | FastEthernet0/0 | 172.16.0.2 | 255.255.224.0 | 172.16.0.1 |
| CEO2 | FastEthernet0/1 | 172.16.0.3 | 255.255.224.0 | 172.16.0.1 |
| Dialer1 | FastEthernet0/2 | 172.16.32.2 | 255.255.224.0 | 172.16.32.1 |
| Dialer2 | FastEthernet0/0 | 172.16.32.3 | 255.255.224.0 | 172.16.32.1 |
| Copywriter1 | FastEthernet0/1 | 172.16.64.2 | 255.255.224.0 | 172.16.64.1 |
| Copywriter2 | FastEthernet0/2 | 172.16.64.3 | 255.255.224.0 | 172.16.64.1 |
| Emp1 | FastEthernet0/0 | 172.16.192.2 | 255.255.224.0 | 172.16.192.1 |
| Emp2 | FastEthernet0/1 | 172.16.192.3 | 255.255.224.0 | 172.16.192.1 |
| guest | FastEthernet0/2 | 172.16.192.4 | 255.255.224.0 | 172.16.192.1 |

Configure R2:

  - Seiral 0/1/0:

En

Conf t

Interface se0/1/0

Ip address 172.16.96.2 255.255.224.0

No shutdown

Exit

  - Seiral 0/1/1:

Interface se0/1/1

Ip address 172.16.160.2 255.255.224.0

No shutdown

Exit

Exit

wr

Configure R3:

  - Seiral 0/1/0:

En

Conf t

Interface se0/1/0

Ip address 172.16.128.2 255.255.224.0

No shutdown

Exit

  - Seiral 0/1/1:

Interface se0/1/1

Ip address 172.16.96.1 255.255.224.0

No shutdown

Exit

  - Gig 0/0/0:

Interface gig0/0/0

Ip address 172.16.192.1 255.255.224.0

No shutdown

Exit

Exit

wr

Configure R1:

  - Seiral 0/1/0:

En

Conf t

Interface se0/1/0

Ip address 172.16.128.1 255.255.224.0

No shutdown

Exit

  - Seiral 0/1/1:

Interface se0/1/1

Ip address 172.16.160.1 255.255.224.0

No shutdown

Exit

  - Gig 0/0/0:

Interface gig0/0/0

No shutdown

Exit

Lab Task 4: Configure R1 for Inter-VLAN Routing
(Listed commands were executed on R1, unless otherwise stated.)

  - Gig0/0/0.10

int g0/0/0.10

encapsulation dot1q 10

ip add 172.16.0.1 255.255.224.0

exit

  - Gig0/0/0.20

int g0/0/0.20

encapsulation dot1q 20

ip add 172.16.32.1 255.255.224.0

exit

  - Gig0/0/0.30

int g0/0/0.30

encapsulation dot1q 30

ip add 172.16.64.1 255.255.224.0

exit

  - On S1-Office1 & S2-Office2, set GigabitEthernet 0/1 as Trunk, with appropriate Native VLAN.

en

conf t

int gig0/1

switchport mode trunk

switchport trunk native vlan 100

no shutdown

exit

Lab Task 5: Static Routing for network devices

  - R3-Office 2:

En

Conf t

ip route 172.16.0.0 255.255.224.0 172.16.128.1

ip route 172.16.32.0 255.255.224.0 172.16.128.1

ip route 172.16.64.0 255.255.224.0 172.16.128.1

ip route 172.16.96.0 255.255.224.0 172.16.96.2

ip route 172.16.160.0 255.255.224.0 172.16.96.2

exit

wr

  - R2-Edge_router:

En

Conf t

ip route 172.16.0.0 255.255.224.0 172.16.160.1

ip route 172.16.32.0 255.255.224.0 172.16.160.1

ip route 172.16.64.0 255.255.224.0 172.16.160.1

ip route 172.16.128.0 255.255.224.0 172.16.96.1

ip route 172.16.192.0 255.255.224.0 172.16.96.1

exit

wr

  - R1-Office 1:

en

conf t

ip route 172.16.96.0 255.255.224.0 172.16.96.2

ip route 172.16.192.0 255.255.224.0 172.16.128.2

ip route 0.0.0.0 0.0.0.0 172.16.160.2

exit

Lab Task 7: Initial and Security Settings for Network Devices
(Listed commands were executed on all routers and switches)

- en
conf t
username Admin password ACDC1973

- line console 0
login local
exit

- enable password beatles1960

- service password-encryption

- banner motd #Access to this device is for authorized personnel only!#
ex
wr
ex

Lab Task 8: Secure Remote Access
(Listed commands were executed on R1, R2, and R3)

- ip domain-name aast.com

- crypto key generate rsa 1024

- ip ssh version 2

- line vty 0 4
login local
motd-banner
transport input ssh
exit

- ex
sho ip ssh
sho run
wr

- Using the command ssh -l Admin IP-Address, I was able to successfully SSH into the routers.